Forest Schools:  Tools safety and tool operating procedures

and Risk Assessment

TOOL SAFETY AND PROCEDURES– BOWSAW

Storage and Transport

- Permanent storage will be in the Forest Schools Leaders store area in a solid tool box. - Transportation to site – Bowsaws will be carried in a secure solid tool box or thick canvas bag ensuring blade guards are on and tools are kept dry. - Access to this bag or tool box will be strictly controlled by the Forest Schools Leader - The Forest Schools Leader is the only person who gets out tools and these will be counted out and counted in whenever used. - Bow saws will be carried holding the bow area like a shopping bag down to the side.

Condition and Maintenance

- Check the frame and any hand guard for cracks or splits - Check that there is a blade guard on the bowsaw - Check the blade mechanisms – that the blade is securely installed and locked in place. - Remove the blade guard - Visually inspect the blade teeth for sharpness, rust and missing teeth - Gently twang the blade to check it is secure and taught - Use regular triangular blade (Peg tooth) for dry, dead wood and an irregular (Raker tooth) blade for green, living wood. - Blade guard returned after use - Following a saw use session, the Forest Schools Leader will use an oily paintbrush to clean the blade and return the blade cover. - When blades become blunt they will be replaced - If the bow saw frame or blade mechanisms become damaged the bow saw will be discarded and replacements purchased.

PPE and Clothing

- Forest Schools Leader to wear safety boots with steel toe caps - Ensure no gloves are worn on the tool hand - Loose clothing and long hair to be secured and tied back - Ensure appropriate outdoor clothing is being worn - Consider use of a high visibility waistcoat by the tool user as a visible warning to other participants

Safe Working Area

- Ensure all site risk assessment checks have been carried out - Consider the level of light and the weather (is there enough light to see clearly, if the weather requires gloves to be worn, don’t use tools, if it is too hot tool users may tire quickly, if it is raining tools more likely to slip so avoid unless working in a sheltered area) - Create a tool using ‘zone’ on the site that the participants know not to run through or play near. Always ensure there is a safe ‘blood bubble’ around the tool users where no-one else can be hurt by the cutting edge of the tool (at least two arms length and length of tool)

Users

- Only introduce a tool when the Forest Schools Leader deems they are ready and knows the participants well – consider the participants previous use or mis-use of tools, ensure they are familiar with the operational procedures above and a 1:1 or 1:2 ratio is possible for initial introductions and practice. - Ensure all fist aid and emergency procedures are known and all facilities accessible. - All adults and Forest School Leader will provide positive role modelling throughout the use of any tool to reassure the participants, and keep the activity safe through any ‘copied’ behaviours and actions.

Use of Tool

- Introduce the tool showing the different parts of the tool and what it can be used for and what it is going to be used for that day. - All use will be continually supervised by the Forest Schools Leader. - Remove blade guard only when ready to saw. Put blade guard in a clearly visible place so it is not lost. Return blade guard if the saw is to be carried. - In between sawing lean the saw up against the A frame - Ensure that there is a ‘safe working area’ or ‘blood bubble’ around the saw users of at least two arms length and the length of the tool and that other participants are aware of where the tool work is taking place so they can avoid the area. - For temporary cutting of a piece of wood use a fork in a tree but as soon as possible replace with an ‘A’ frame or for more permanent sites can create a ‘saw horse’. This raises up the wood and moves the blade away from the feet and legs of the saw user. - Dominant hand holds the tool and the tool user takes a wide secure stance - The non-dominant hand holds the wood on the other side of the ‘V’ in the A frame. - A second person holds the wood to support at the other end of the A frame - No glove on tool hand (better with no gloves at all) - Draw the blade three or four times towards self to make a groove in the wood using the full length of the blade then saw continually - Can be done in pairs with one on either side of the wood if desired but non-dominant hands remain on the other side of the V of the A frame. - Any faults identified during tool use are to be immediately notified to the Forest Schools Leader who will undertake maintenance or remove from use and label immediately. - Return tool to storage container as soon as finished working with it and checked back in by Forest Schools Leader. - As a participant improves in skill and competency the procedures above will be reviewed and it may be possible to adjust to enable tasks to be undertaken more independently – this will depend on the age and individual assessment of the participant and the final decision will lie with the Forest School Leader.

TOOL SAFETY AND PROCEDURES– FIXED BLADE KNIFE

Storage and Transport

- Permanent storage will be in the Forest Schools Leaders store area in a solid tool box. - Transportation to site: The knives will be carried in a solid tool box or similar solid container with the sheaths on ensuring they are kept dry. - Access to tool or storage box will be strictly controlled by the Forest Schools Leader - The Forest Schools Leader is the only person who gets out tools and these will be counted out and counted in whenever used. - Knives will be carried holding the handle with the blade pointing downwards with the sheaths on.

Condition and Maintenance

- Handle secure and intact - Blade sheath intact and not cracked - Check blade for kinks and cracks - Forest Schools Leader check prior to session for sharpness by gently dragging finger across the blade at right angles. - Clearly mark any knives not to be used and put them away. - Following knife use the user can wipe the blade on the wood that has been cut to remove any surplus sap or shavings. - Following the session the Forest Schools Leader will use an oily rag to wipe the blades clean and dry them and then return the sheath. - When blades become blunt or if handles become insecure or damaged they will be taken to a professional for sharpening and replacement or discarded and no longer used.

PPE and Clothing

- Forest Schools Leader to wear safety boots with steel toe caps - Ensure no gloves are worn on the tool hand - Loose clothing and long hair to be secured and tied back - Ensure appropriate outdoor clothing is being worn - Consider use of a high visibility waistcoat by the tool user as a visible warning to other participants

Safe Working Area

- Ensure all site risk assessment checks have been carried out - Consider the level of light and the weather (is there enough light to see clearly, if the weather requires gloves to be worn, don’t use tools, if it is too hot tool users may tire quickly, if it is raining tools more likely to slip so avoid) - Create a tool using ‘zone’ on the site that the participants know not to run through or play near. Always ensure there is a safe ‘blood bubble’ around the tool users where no-one else can be hurt by the cutting edge of the tool (at least two arms length and length of tool)

Users

- Only introduce a tool when the Forest Schools Leader deems they are ready and knows the participants well – consider the participants use or mis-use of tools, ensure they are familiar with the operational procedures above and a 1:1 or 1:2 ratio is possible for initial introductions and practice. - Ensure all fist aid and emergency procedures are known and all facilities accessible. - All adults and Forest School Leader will provide positive role modelling throughout the use of any tool to reassure the participants, and keep the activity safe through any ‘copied’ behaviours and actions.

Use of Tool

- Introduce the tool showing the different parts of the tool and what it can be used for and what it is going to be used for that day. - All use will be continually supervised by the Forest Schools Leader. - Ensure that there is a ‘safe working area’ or ‘blood bubble’ around the knife users of at least two arms length and the length of the tool and that other participants are aware of where the tool work is taking place so they can avoid the area. - Crouch in a secure and stable three point stance. - Remove the knife sheath and put it somewhere nearby and clearly visible - No glove on the tool hand (better no gloves at all) - Hold the knife firmly - Always work away from the body and away from the legs - For stripping bark from live wood hold the knife at right angles to the wood and scrape away. - For creating a point angle the blade and work away from the body removing a small amount at a time. - When work is completed the knife can be wiped on the ‘clean’ wood area (never with hands or on clothes) and the sheath replaced - Any faults identified during tool use are to be immediately notified to the Forest Schools Leader who will undertake maintenance or remove from use and label immediately. - Return tool to storage container as soon as finished working with it and checked back in by Forest Schools Leader. - As a participant improves in skill and competency the procedures above will be reviewed and it may be possible to adjust to enable tasks to be undertaken more independently – this will depend on the age and individual assessment of the participant and the final decision will lie with the Forest School Leader.

TOOL SAFETY AND PROCEDURES – LOPPERS ANVIL OR BYPASS

Storage and Transport

- Permanent storage will be in the Forest Schools Leaders store area in a solid tool box. - Transportation to site – Loppers will be carried in a secure solid tool box or thick canvas bag ensuring tools are kept dry. - Access to this bag or tool box will be strictly controlled by the Forest Schools Leader - The Forest Schools Leader is the only person who gets out tools and these will be counted out and counted in whenever used. - Loppers will be carried with the blades pointing downwards holding the lower of the handles to prevent them swinging open when being carried. Pass with the handles.

Condition and Maintenance

- Check the handles for any cracks or splits and that they are attached properly. - Visually check the blades whilst closed for any splits, cracks, rust or damage and check the pivot point is a smooth action. - Before the session the Forest Schools Leader will check the sharpness by opening up the loppers bracing the handles against the groin/leg and shoulder so they cannot be caught and shut – drag a finger gently across the blade at right angles to feel the snag and check they are sharp. - Following lopper use (and using the same stance as above to ensure they cannot be swung shit) an oily rag will be used to wipe the blades clean and then dried prior to storage. - When blades become blunt or if handles become insecure or damaged they will be taken to a professional for sharpening and replacement or discarded and no longer used.

PPE and Clothing

- Forest Schools Leader to wear safety boots with steel toe caps - Ensure no gloves are worn on the tool hand - Loose clothing and long hair to be secured and tied back - Ensure appropriate outdoor clothing is being worn - Consider use of a high visibility waistcoat by the tool user as a visible warning to other participants. - If long pieces of wood are being lopped which may be above head height the tool users and those assisting to wear a hard hat (these will be checked prior to use to ensure there are no cracks or damage, that they are in date for use and that they fit properly)

Safe Working Area

- Ensure all site risk assessment checks have been carried out - Consider the level of light and the weather (is there enough light to see clearly, if the weather requires gloves to be worn, don’t use tools, if it is too hot tool users may tire quickly, if it is raining tools more likely to slip so avoid) - Create a tool using ‘zone’ on the site that the participants know not to run through play near. Always ensure there is a safe ‘blood bubble’ around the tool users where no-one else can be hurt by the cutting edge of the tool (at least two arms length and length of tool)

Users

- Only introduce a tool when the Forest Schools Leader deems they are ready and knows the participants well – consider the participants previous use or mis-use of tools, ensure they are familiar with the operational procedures above and a 1:1 or 1:2 ratio is possible for initial introductions and practice. - Ensure all fist aid and emergency procedures are known and all facilities accessible. - All adults and Forest School Leader will provide positive role modelling throughout the use of any tool to reassure the participants, and keep the activity safe through any ‘copied’ behaviours and actions.

Use of Tool

- Introduce the tool showing the different parts of the tool and what it can be used for and what it is going to be used for that day. - All use will be continually supervised by the Forest Schools Leader. - Loppers need to be carried safely – establish that tool users identify where and what they want to harvest and come back for the Forest Schools Leader and the loppers so carrying can be supervised. - Carry the loppers as dictated above to the place where they are to be used (support tool users in selection of materials to be harvested – type and size of wood) Ensure the wood is of a suitable diameter for the loppers (around thumb thickness or 2 pence piece maximum – no thicker than the diameter of the handles) and the cut is made in a healthy place for the tree. - Open up the loppers and place the blades around the wood to be harvested in the correct place and push the blades together. Never put any part of the body between the lopper jaws. - Ensure the tool user is not overloaded so one carries the tool and another carries the material. - Any faults identified during tool use are to be immediately notified to the Forest Schools Leader who will undertake maintenance or remove from use and label immediately. - Return tool to storage container as soon as finished working with it and checked back in by Forest Schools Leader. - As a participant improves in skill and competency the procedures above will be reviewed and it may be possible to adjust to enable tasks to be undertaken more independently – this will depend on the age and individual assessment of the participant and the final decision will lie with the Forest School Leader.

TOOL SAFETY AND PROCEDURES – FOLDING SAW

Storage and Transport

- Permanent storage will be in the Forest Schools Leaders store area in a solid tool box. - Transportation to site – Folding saws will be carried in a secure solid tool box or thick canvas bag ensuring the blades are closed and tools are kept dry. - Access to this bag or tool box will be strictly controlled by the Forest Schools Leader - The Forest Schools Leader is the only person who gets out tools and these will be counted out and counted in whenever used. - Folding saws will be carried in a closed position holding the handle firmly and pointing downwards.

Condition and Maintenance

- Check the handle for any cracks, splits or damage - Check the opening mechanism and that the blade is securely installed - Carefully open the blade and ensure that it securely locks open. - Visually inspect the blade teeth for sharpness - As soon as saw use is complete close the saw carefully using the mechanism and holding the back of the blade. - Following a saw use session the Forest Schools Leader will use an oily paintbrush to clean the blade. - When blades become blunt they will be replaced - If the handle or folding mechanism become damaged the saw will be discarded and replacements purchased.

PPE and Clothing

- Forest Schools Leader to wear safety boots with steel toe caps - Ensure no gloves are worn on the tool hand - Loose clothing and long hair to be secured and tied back - Ensure appropriate outdoor clothing is being worn - Consider use of a high visibility waistcoat by the tool user as a visible warning to other participants - If long pieces of wood are being lopped which may be above head height the tool users and those assisting to wear a hard hat (these will be checked prior to use to ensure there are no cracks or damage, that they are in date for use and that they fit properly)

Safe Working Area

- Ensure all site risk assessment checks have been carried out - Consider the level of light and the weather (is there enough light to see clearly, if the weather requires gloves to be worn, don’t use tools, if it is too hot tool users may tire quickly, if it is raining tools more likely to slip so avoid) - Create a tool using ‘zone’ on the site that the participants know not to run through play near. Always ensure there is a safe ‘blood bubble’ around the tool users where no-one else can be hurt by the cutting edge of the tool (at least two arms length and length of tool)

Users

- Only introduce a tool when the Forest Schools Leader deems they are ready and knows the participants well – consider the participants use or mis-use of tools, ensure they are familiar with the operational procedures above and a 1:1 or 1:2 ratio is possible for initial introductions and practice. - Ensure all fist aid and emergency procedures are known and all facilities accessible. - All adults and Forest School Leader will provide positive role modelling throughout the use of any tool to reassure the participants, and keep the activity safe through any ‘copied’ behaviours and actions.

Use of Tool

- Introduce the tool showing the different parts of the tool and what it can be used for and what it is going to be used for that day. - All use will be continually supervised by the Forest Schools Leader. - Folding saws need to be carried safely – establish that tool users identify where and what they want to harvest and come back for the Forest Schools Leader and the folding saw so carrying can be supervised. - Carry the folding saw as dictated above to the place where they are to be used (support tool users in selection of materials to be harvested – type and size of wood) Ensure the wood is of a suitable diameter for the loppers (up to wrist width) and the cut is made in a healthy place for the tree. - Open up the folding saw – assistants to support the wood if necessary ensuring that the blade always cuts away from the person. - To cut thicker wood carry out a ‘step cut’ – Cut about two thirds of the way through the trunk (in the direction you want the wood to fall) on the opposite side cut about a third of the way through the trunk and help the wood to fall leaving a ‘step’ in the trunk. - Lower and move the harvested wood to a safe place. - Saw straight across the sawn wood as close to the base as possible to tidy it up, use some of the soil/mud around the area to seal the wood and disguise the harvested area. - Ensure the tool user is not overloaded so one carries the tool and another carries the material or the harvested wood is dragged carefully or bundled effectively so participants don’t have to lift the materials. - Any faults identified during tool use are to be immediately notified to the Forest Schools Leader who will undertake maintenance or remove from use and label immediately. - Return tool to storage container as soon as finished working with it and checked back in by Forest Schools Leader. - As a participant improves in skill and competency the procedures above will be reviewed and it may be possible to adjust to enable tasks to be undertaken more independently – this will depend on the age and individual assessment of the participant and the final decision will lie with the Forest School Leader.

TOOL SAFETY AND PROCEDURES – AXE

Storage and Transport

- Permanent storage will be in the Forest Schools Leaders store area in a solid tool box. - Transportation to site – Axe will be carried in a secure solid tool box or thick canvas bag ensuring blade cover is on and tools are kept dry. - Access to this bag or tool box will be strictly controlled by the Forest Schools Leader - The Forest Schools Leader is the only person who gets out tools and these will be counted out and counted in whenever used. - Axes will be carried holding the handle securely and pointing the blade downwards and to the side of the body.

Condition and Maintenance

- Check the handle for any cracks or splits - Check the blade is securely fitted to the handle (check the fixings in the top of the axe) - Remove the blade cover and visually inspect the blade for any kinks, cracks or rust - Forest Schools Leader to check for blade sharpness before the session starts by holding the axe away from the body and gently dragging a finger across the blade at right angles to feel for a ‘snag’. - Blade cover returned after use - Following an axe use session the Forest Schools Leader will use an oily rag to wipe and dry the blade and replace the blade cover. - When blades become blunt they will be taken to a professional to be sharpened. - If the handle or blade become damaged or loose they will be sent to a professional for servicing in line with product guarantees. - Ensure that the axe is used onto an axe ‘block’ of other wood to prevent the blade becoming damaged or causing problems by cutting onto rock, stone or other metal.

PPE and Clothing

- Forest Schools Leader to wear safety boots with steel toe caps - Ensure no gloves are worn on the tool hand - Loose clothing and long hair to be secured and tied back - Ensure appropriate outdoor clothing is being worn - Consider use of a high visibility waistcoat by the tool user as a visible warning to other participants

Safe Working Area

- Ensure all site risk assessment checks have been carried out - Consider the level of light and the weather (is there enough light to see clearly, if the weather requires gloves to be worn, don’t use tools, if it is too hot tool users may tire quickly, if it is raining tools more likely to slip so avoid) - Create a tool using ‘zone’ on the site that the participants know not to run through play near. Always ensure there is a safe ‘blood bubble’ around the tool users where no-one else can be hurt by the cutting edge of the tool (at least two arms length and length of tool)

Users

- Only introduce a tool when the Forest Schools Leader deems they are ready and knows the participants well – consider the participants use or mis-use of tools, ensure they are familiar with the operational procedures above and a 1:1 or 1:2 ratio is possible for initial introductions and practice. - Ensure all fist aid and emergency procedures are known and all facilities accessible. - All adults and Forest School Leader will provide positive role modelling throughout the use of any tool to reassure the participants, and keep the activity safe through any ‘copied’ behaviours and actions.

Use of Tool

- Introduce the tool showing the different parts of the tool and what it can be used for and what it is going to be used for that day. - All use will be continually supervised by the Forest Schools Leader. - Remove the blade cover and put it in a clearly visible place nearby. - When the axe is not in use temporarily embed it in the axe block so it is not loose or overhanging the edge of the axe block. For any period of non-use put the blade cover on and put the tool away. - Use a three point stance next to the axe block. - For splitting wood – participant places tool in the correct position (other participant holds the wood) and is told when to tap the axe blade into the wood (ensure this is of an appropriate thickness and length) and use a mallet to gently tap the blade into wood until it splits – only taps when told to do so by axe user. When the axe is firmly embedded it can be tapped into the wood further to split longer pieces of wood. - To create spikes – use a partner to hold the end of the log and lay the wood flat across the axe block. Tool user to hold the wood in the non-dominant hand cupping underneath the log. - Bring the axe down onto the bark of the wood making small ‘chipper cuts’ to break the grain of the wood. Rotate the wood until there are chipper cuts all the way around. - The partner now to hold up the end of the wood so it is on an angle. The axe always cuts vertically downwards so ensure that the wood is angled not the axe angle. Bring down the axe rotating the wood gradually cutting to create a point (the tool user gives instruction to the wood holder when they want a rotation or an angle change) ensure that the non-dominant hand is well away from the cutting area and cupped underneath the wood – not on top. - Any faults identified during tool use are to be immediately notified to the Forest Schools Leader who will undertake maintenance or remove from use and label immediately. - Return tool to storage container as soon as finished working with it and checked back in by Forest Schools Leader. - As a participant improves in skill and competency the procedures above will be reviewed and it may be possible to adjust to enable tasks to be undertaken more independently – this will depend on the age and individual assessment of the participant and the final decision will lie with the Forest School Leader.

TOOL SAFETY AND PROCEDURES – MALLET

Storage and Transport

- Permanent storage will be in the Forest Schools Leaders store area in a solid tool box. - Transportation to site – Mallet will be carried in a secure solid tool box or thick canvas bag ensuring tools are kept dry. - Access to this bag or tool box will be strictly controlled by the Forest Schools Leader - The Forest Schools Leader is the only person who gets out tools and these will be counted out and counted in whenever used. - Mallet to be carried holding the handle and pointing downwards.

Condition and Maintenance

- Check the handle and striking end for any cracks, breakages or damage - Ensure that the striking end is secure on the handle - If the mallet becomes damaged it will be discarded and replaced.

PPE and Clothing

- Forest Schools Leader to wear safety boots with steel toe caps - Ensure no gloves are worn on the tool hand - Loose clothing and long hair to be secured and tied back - Ensure appropriate outdoor clothing is being worn - Consider use of a high visibility waistcoat by the tool user as a visible warning to other participants

Safe Working Area

- Ensure all site risk assessment checks have been carried out - Consider the level of light and the weather (is there enough light to see clearly, if the weather requires gloves to be worn, don’t use tools, if it is too hot tool users may tire quickly, if it is raining tools more likely to slip so avoid) - Create a tool using ‘zone’ on the site that the participants know not to run through play near. Always ensure there is a safe ‘blood bubble’ around the tool users where no-one else can be hurt by the cutting or striking edge of the tool (at least two arms length and length of tool)

Users

- Only introduce a tool when the Forest Schools Leader deems they are ready and knows the participants well – consider the participants use or mis-use of tools, ensure they are familiar with the operational procedures above and a 1:1 or 1:2 ratio is possible for initial introductions and practice. - Ensure all fist aid and emergency procedures are known and all facilities accessible. - All adults and Forest School Leader will provide positive role modelling throughout the use of any tool to reassure the participants, and keep the activity safe through any ‘copied’ behaviours and actions.

Use of Tool

- Introduce the tool showing the different parts of the tool and what it can be used for and what it is going to be used for that day. - All use will be continually supervised by the Forest Schools Leader. - Ensure that the item that is to be hit with the mallet (peg, log, axe, etc.) is held securely by the tool user and hands are away from the area to be struck - Ensure no-one is in the ‘swing area’ and outside the tool use ‘blood bubble’ - Hit the item cleanly and firmly but don’t over swing and ensure concentration to connect the mallet with the object. - Any faults identified during tool use are to be immediately notified to the Forest Schools Leader who will undertake maintenance or remove from use and label immediately. - Return tool to storage container as soon as finished working with it and checked back in by Forest Schools Leader. - As a participant improves in skill and competency the procedures above will be reviewed and it may be possible to adjust to enable tasks to be undertaken more independently – this will depend on the age and individual assessment of the participant and the final decision will lie with the Forest School Leader.

| Risk Assessment for Tool Use at Forest Schools |  |  |  |  |  | Reviews |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Task/Operation: | Tool use at Forest Schools – Bowsaws, Loppers, Axe, Fixed Blade Knife, Mallet, Folding Saw |  |  |  |  | Next review needed | Date actually reviewed | Changes needed? (Yes/No) |  | Assessor initials |  |  |
| Group(s) affected: | Teachers, TAs, school staff, adult helpers, pupils, public |  |  |  |  |  |  |  |  |  |  |  |
| Person(s) undertaking assessment: | Rachael Muir |  |  |  |  |  |  |  |  |  |  |  |
| Employee(s) consulted: | Derbyshire LA risk assessments and school risk assessments |  |  |  |  |  |  |  |  |  |  |  |
| Union Safety Rep(s) consulted: | No |  |  |  |  |  |  |  |  |  |  |  |
| Date of assessment: | May 2019 |  |  |  |  |  |  |  |  |  |  |  |
| PLEASE NOTE – THIS RISK ASSESSMENT WORKS IN CONJUNCTION WITH THE SITE AND GROUP RISK ASSESSMENT AND TOOL SAFETY AND PROCEDURE DOCUMENTS. ALL PROCEDURES, SITE RISKS, GROUP RISKS, FIRST AID AND EMERGENCY PROCEDURES WILL BE FOLLOWED IN LINE WITH THOSE ARRANGMENTS. |  |  |  |  |  |  |  |  |  |  |  |  |
| L (Likelihood) X S (Severity) = RR (Risk Rating) 1 is the lowest 3 is highest |  | # Before control |  |  | FSL – Forest Schools Leader; SA – Supervising Adult; P - Participants |  |  |  | # After control |  |  |  |
| Hazard | Resultant effect(s) | L | # S | R R | State control measures to be put in place and by whom |  |  |  | # L |  | # S | RR |
| Injury from incorrect technique and tool use | Cut, abrasion, injury to user or those nearby, strain injury | 2 | 3 | 6 | - Participants and staff trained in safe tool handling and use prior to any activity. Also trained in body positioning for user and others and safe distances. (FSL) - Tool use supervised at all time. Activities stopped if safety of participants is compromised (FSL) - Correct PPE wear available at all times and used correctly. (FSL, SA, P) - Tool use areas away from other work areas (FSL, SA, P) - First aid kit, first aider and emergency procedures to be followed (FSL, SA, P) - Manual handling of tools and harvested materials to be supervised (FSL) - Access to tools controlled by Forest Schools Leader and counted in and out at all times. |  |  |  | 1 |  | 3 | 3 |
| Unsafe handling due to lack of concentration/ boredom/ fatigue | Injury, abrasions, cuts, strains to user or those nearby | 2 | 3 | 6 | - Tool use to be supervised by Forest School leader at all times. Small groups worked with. Forest School leader will stop the activity if safety of participants is being compromised. (FSL) - Energy flow of group to be monitored at all times to ensure activity does not take too long. Rotation of tasks and responsibilities to keep interest and motivation. (FSL) - Regular rest periods to be built in (FSL) - PPE wear available at all times. Operators must wear appropriate PPE gear when using tools. This is covered in the tool introduction (FSL, SA, P) - Tool use in an area away from other work areas (FSL, SA, P) - First aid kit, first aider and emergency procedures to be followed (FSL, SA, P) |  |  |  | 1 |  | 3 | 3 |
| Unsafe handling due to weather conditions | Cuts, abrasions, strains, injury to user or those nearby | 2 | 3 | 6 | - Forest School leader to check weather forecasts before the forest schools sessions and not use tools or end tool use if weather deteriorates and use of tools becomes unsafe (FSL) - Correct PPE wear available at all times. Tool users must wear appropriate PPE gear when using the tool (FSL, SA, P) - Tool use in an area away from other work areas (FSL, SA, P) - First aid kit, first aider and emergency procedures to be followed (FSL, SA, P) |  |  |  | 1 |  | 3 | 3 |
| Injury from the use of poorly maintained tools | Fatigue, cuts, strain, abrasion, injury to user or those nearby | 2 | 3 | 6 | - Tool condition to be checked prior to use by Forest School leader (FSL) - All tools to be regularly maintained and checked Faulty items to be removed and labelled until repaired/ disposed of (FSL) - Participants and staff advised as part of tool introduction to alert the Forest School leader immediately if they have any concerns about the tools they are using so they can be removed or repaired (FSL, SA, P) - First aid kit, first aider and emergency procedures to be followed (FSL, SA, P) |  |  |  | 1 |  | 3 | 3 |